- Primary Information

| Sample ID | R/2024050001 A |
| --- | --- |
| Date Received | 30 Mei 2024 |
| Quotation ID | Q-2405-099 |
| Turn Around Time | 10 Juni 2024 |

- Customer Information

| Customer Name: | PT. Contoh Pelanggan Sejahtera |
| --- | --- |
| Contact Person: | Bpk. Budi |
| Mobile: | 0812345665 |
| Email: | example@example.com |
| Address: | Jl Merdeka 19, Jakarta, Indonesia |

- Sample Details

| Sample Photo |  |
| --- | --- |
| Matrix / Sample Type: | Air Limbah Domestik (Domestic Wastewater) |
| Quantity & Unit: | 2 x 1 Liter Botol Kaca |
| Storage Location: | Kulkas 1 - Rak B (Chiller 4°C) |
| Condition Upon Receipt: | Lainnya: Suhu saat diterima 6°C. Segel sedikit terbuka. |

- Sampling

- Sampling Data

| Sampling Date & Time: | 29 Mei 2024, 09:30 WIB |
| --- | --- |
| Sampling Location: | Outlet IPAL - Titik Koordinat: -6.2000, 106.8166 |
| Sampler Name: | Ahmad Junaedi (Petugas Lab) / Customer |
| Sampling Method: | ☑ Grab (Sesaat) ☐ Composite (Gabungan) |
| Weather Condition: | Cerah / Berawan / Hujan |

- Field Measurements

| Parameter | Result | Unit | Method/Alat |
| --- | --- | --- | --- |
| Temperature (Suhu) | 28.5 | °C | Thermometer |
| pH (Lapangan) | 7.2 | - | pH Meter Portable |
| DO (Dissolved Oxygen) | 5.4 | mg/L | DO Meter |
| Debit (Flow) | 1.5 | m³/s | Flowmeter |

- Requested Test

| No | Parameter Uji | Sub-Parameter | Metode | Alat / Instrumen | Due Date |
| --- | --- | --- | --- | --- | --- |
| 1 | Logam Berat | Arsen (As) | SNI 6989.82:2018 | ICP-OES | 02/06/2024 |
| 2 | Logam Berat | Kadmium (Cd) | SNI 6989.82:2018 | ICP-OES | 02/06/2024 |
| 3 | Logam Berat | Kromium (Cr-VI) | SNI 6989.82:2018 | ICP-OES | 02/06/2024 |
| 4 | Logam Berat | Tembaga (Cu) | SNI 6989.82:2018 | ICP-OES | 02/06/2024 |
| 5 | Logam Berat | Besi (Fe) | SNI 6989.82:2018 | ICP-OES | 02/06/2024 |
| 6 | Logam Berat | Timbal (Pb) | SNI 6989.82:2018 | ICP-OES | 02/06/2024 |
| 7 | Logam Berat | Mangan (Mn) | SNI 6989.82:2018 | ICP-OES | 02/06/2024 |
| 8 | Logam Berat | Merkuri (Hg) | SNI 6989.82:2018 | ICP-OES | 02/06/2024 |
| 9 | Logam Berat | Seng (Zn) | SNI 6989.82:2018 | ICP-OES | 02/06/2024 |
| 10 | Logam Berat | Nikel (Ni) | SNI 6989.82:2018 | ICP-OES | 02/06/2024 |
| 11 | Logam Berat | Selenium (Se) | SNI 6989.82:2018 | ICP-OES | 02/06/2024 |
| 12 | Logam Berat | Kobalt (Co) | SNI 6989.82:2018 | ICP-OES | 02/06/2024 |
| 13 | pH | - | SNI 06-6989.11-2004 | pH Meter Mettler | 31/05/2024 |
| 14 | COD | - | SNI 6989.2:2019 | UV-Vis Spectro | 02/06/2024 |
| 15 | TSS | - | APHA 2540 D | Analytical Balance | 02/06/2024 |

Assigned By:

Supervisor,

Farrel Arya Andhika